%N uvtsub

%D Subtract time-averaged "background" visibilities

%P pjt

%: uv analysis

%B

UVTSUB subtracts a time-averaged background level from each

baseline.

WARNING: This program does not apply gains. Since gain corrections

are time-dependent, they must be applied to the data

before subtraction is carried out, or else subtraction

will not work correctly.

%A vis

The name of the input uv data-set. No default.

%A out

The name of the output uv data-set. No default.

%A select

The normal uv selection commands. The default is copy everything.

%A bckgrnd

The time interval used to determine the average visibilities

for subtraction. This must be in the standard select= format,

e.g.

bckgrnd=time(12:15:30,12:18:20),time(12:25:30,12:26:00)

would select two intervals to be used to determine the

"background" level for subtraction.