%N xfcor

%D Model correlator configuration for CARMA.

%P mchw

%: utility

%B

XFCOR is a MIRIAD task to model the CARMA correlator configuration.

Input an observing frequency and IF freq and search

spectral line files for all spectral lines that would be

observable in IF passband. The positions of the lines in

the correlator for the selected correlator settup are plotted.

%A in

The name of the spectral line file.  Up to 5 input files are

permitted.  Available files are: $MIRCAT/lovas.3mm, $MIRCAT/lovas.1mm

and $MIRCAT/recom.lis. The default is $MIRCAT/lovas.3mm.

%A freq

The rest frequency of the line in GHz. The default 110.0 GHz.

%A iffreq

The intermediate frequency in MHz. The default 1250.0 MHz.

This is the frequency in the IF at which the line

rest frequency will appear. A negative value indictes that

the line should appear in the lower sideband of the first LO.

%A vlsr

Vlsr of the source in km/s.  The default 0 km/s.

%A device

The device used for plotting.  See the User's Manual for

details on how to specify the PGPLOT device.

%A log

The output log file.  The default is the terminal.

%A cormode

The mode determines the correlator configuration and number of

spectral windows. Default=3. The number of channels corresponds

to Table 1 in CARMA memo 28.

%A corf

Correlator LO frequencies in MHz.

Three values. One for each frequency band.

Default corbw=1000,1500,2000

%A corbw

Correlator bandwidths in MHz. Three values selected from the

available bandwidths: 500, 62, 31, 8, or 2.

Default corbw=500,500,500

%A birdie

Number of birdies followed by a list of the birdie frequencies

in MHz. Up to 20 birdies are allowed. The default is 0 birdies.