%N tmgen

%D Simulate a SMA polarimetric observation.

%P rjs

%: uv analysis, map making

%B

tmgen forms a visibility dataset which has some of the characteristics

of an SMA polarimetric dataset. In particular it can optionally produce

time-multiplexed polarization products and two-part, Nasmyth-like leakages.

%A vis

The input visibility dataset. No default. NOTE: For a number of the

steps performed by this task, the input visibility dataset must

have four simultaneous polarization correlations (e.g. XX, YY, XY, YX

or RR, LL, RL, LR).

%A out

The output visibility dataset. No default.

%A polar

Polarization patterns for generating time shared polarization data.

Up to MAXPOLAR=100 strings of the characters R and L, or X and Y,

to represent the polarization of each antenna

R, L (right and left circular polarization), X and Y (linear

polarization).

E.g. for 3 antennas, the polar=LLL,LRR,RRL,RLR cycles

through all combinations of LCP and RCP for each baseline every

4 integrations. The default is to not subset the data based

on polarization.

%A leakage

Simulate polarimetric leakage. This parameter gives the leakage magnitude

as a percent. Random leakages with this rms value are generated and

applied to the data.

%A options

Extra processing options.

nasmyth   Add an extra rotation to the polarisations that would result

from SMA-style Nasmyth receiver placement.